REQUEST FOR PROPOSAL (RFP)

RFP Reference: RFP – 266125
Issue Date: 20/03/2026
Issued by: Ucom CJSC

LOT 5

- INTRODUCTION

UCOM CJSC is requesting proposals from qualified vendors for the supply of hardware equipment as outlined in this document. The purpose of this RFQ is to solicit competitive bids for the procurement of high-quality and cost-effective hardware solutions that meet our operational needs.

- BACKGROUND INFORMATION

UCOM CJSC is the fastest fixed and mobile services provider in Armenia. The company is the absolute leader of the local IPTV and fixed internet market, as well as occupies the key position on the Armenian mobile internet market.

- SCOPE OF WORK The selected vendor shall provide the following hardware components according to the part numbers and quantity mentioned in the BoM below:

  - General Requirements

Two (2) enterprise storage systems must support the following key features:

- Data protection mechanisms including RAID 1, RAID 5, RAID 6, RAID 10 or equivalent distributed parity or erasure coding schemes

- Thin provisioning support

- Snapshot functionality for point-in-time data protection

- Automated storage tiering and/or SSD caching

- Asynchronous replication

  - Hardware Configuration

    - Controllers

- Minimum 2 hot-swappable Active-Active controllers

- Multi-core server-class processors

- Minimum ~32 GB total system memory (~16 GB per controller)

- Scalable controller architecture

  - Disk Subsystem

Chassis:

- 24 × 2.5″ SFF hot-swappable drive bays

- Expandable up to ≥276 drives via expansion enclosures (or similar)

Supported Drive Types (minimum):

- SAS SSD

- SAS HDD 10K

- NL-SAS 7.2K

- Optional support for encrypted drives (SED/FIPS certified)

  - Connectivity / Host Interfaces

Required host ports:

- Fibre Channel: 16/32 Gb/s

- iSCSI: 10/25 Gb/s

Additional:

- Management Ethernet port ≥1 Gb

- Optional additional ports/adapters

  - Software Features & Support

- 3 Years warranty from the manufacturer

System must support:

- Monitoring and alerts (SNMP/Email)

- Data protection at RAID and logical level

Supported OS / Platforms:

- Microsoft Windows Server

- Linux (RHEL, SLES, others)

- VMware ESXi

- Citrix XenServer

  - Power and Environmental Requirements

Power Supply:

- Dual redundant (e.g., 580 W or equivalent), hot-swappable

- Auto-ranging support

Cooling: Sufficient rack operation with automatic fan management

Operating Conditions:

- Temperature: 5 °C – 35 °C

- Humidity: 8 % – 85 % non-condensing

  - Disk Capacity

- 9x7.68TB SAS SSD

  - Spare parts

- 4 x FC 16/32Gbps transceiver for this storage

- 4 x ISCSI 10/25Gb transceiver for this storage

4. VENDOR QUALIFICATIONS Vendors responding to this RFP must meet the following requirements:

- Proven experience in hardware procurement and deployment

- Ability to provide competitive pricing and bulk order discounts

- Strong post-sale support, including warranty

- Compliance with industry standards and regulations

5. PROPOSAL REQUIREMENTS Proposals should include the following:

- Company Information: Name, address, contact details, and relevant certifications.

- Technical Proposal: Detailed specifications of the proposed hardware, compatibility, and any additional features complying with the provided BoM.

- Pricing Proposal: Unit cost, bulk discounts, and total cost, including taxes and shipping.

- Warranty & Support: Details on warranty coverage

- Delivery Schedule: Estimated lead time for delivery and installation.

6. SELECTION CRITERIA Proposals will be evaluated based on the following criteria:

- Compliance with technical specifications (must comply)

- Pricing competitiveness (100%)

7. RFP TIMELINE

- RFP Issued: [Date]

- Questions Submission Deadline: [Date]

- Proposal Submission Deadline: [Date]

- Vendor Selection Announcement: [Date]

- Contract Signing and Procurement Start: [Date]

8. SUBMISSION GUIDELINES All proposals must be submitted in electronic format to [Email Address] or mailed to [Company Address] no later than [Deadline Date]. Late submissions will not be considered.

For any inquiries regarding this RFP, please contact [Contact Name] at [Phone Number] or [Email Address].

9. TERMS & CONDITIONS UCOM CJSC reserves the right to accept or reject any proposal, negotiate terms, and make an award in the best interest of the company. Vendors are responsible for all costs incurred in the preparation of the proposal.

### CONTACTS FOR CLARIFICATIONS

For commercial inquiries, contacts:
Department: Procurement & Logistics
Name: Armen Mikayelyan ; Georgi Arakelyan
Email:  Armen.Mikayelyan@ucom.am ; Georgi.Arakelyan@ucom.am 
Application acceptance deadline: 03 April, 2026

END OF RFP